(You can make this Help File stay on top of other windows by clicking

Options | Stay On Top)

APRS-Beacon

What is APRS-Beacon?

APRS-Beacon is a 32-bit MS-Windows program. It has been tested on Windows 95, 98se and ME.

The purpose of the APRS-Beacon is to provide simple APRS-compatible position beacons for up to three Objects. It is designed to 'stand alone' and does not monitor other traffic on the frequency.    It can 'talk'    to a single-port TNC in standard ('Vanilla') AX25 mode, a single or dual port TNC in KISS mode, or the AGW Packet Engine [1] when it can transmit on up to four radio ports.    (If you will only be using AGWPE, APRS-Beacon will also run on Windows 2000).

Each Object allows for the transmission of an optional 'comment' field of up to 40 characters. This field can be automatically updated in real time by obtaining the comment from a simple external text file.    The format of this file is shown at the end of this Help file.

Object 1 (only) can be used to transmit weather information in standard APRS format instead of an Object.

The thinking behind writing APRS-Beacon was to provide a simple way for event-organisers (of rallies, for example) to set up a quick APRS position beacon without necessarily having any knowledge of    APRS.

Installing APRS-Beacon

Installation simply consists of unzipping  APRSBeacon.exe and APRSBeacon.doc into any folder of your choice and creating a Desktop Shortcut to the exe file. Also included in the zip package is a sample Map file (NorthWest.gif) and its associated information file (NorthWest.inf).    These can be unzipped into the same folder as APRSBeacon.exe    or to any other folder of your choice.

Running APRS-Beacon for the first time

The APRS-Beacon screen is divided into two sections. The right-hand section will display a map (when you select it) and the left-hand section shows three 'Tabbed' pages showing the details of each of the three Objects (The Details window).

Before selecting a Map, you should click on the Setup Menu and select Communications. Select the 'Comms Type' ( AGWPE, KISS or TNC).    In the case of AGWPE, set the 'Host IP Address'.    The default value is 127.0.0.1 which is normally the correct setting.    The TCP Port value is fixed at 8000 by AGWPE and this should not be changed.

For KISS mode and TNC's, set the COM Port and its associated parameters (Baud Rate, etc).    If    TNC is selected, APRSBeacon needs the TNC to be in 'native' AX25 mode.

For more information about configuring APRS-Beacon for use with a TNC, see Configuring APRS-Beacon for your TNC at the end of this Help file.

Also under the Setup Menu you need to select Station Setup and enter your callsign and SSID.    If    you're using AGWPE and it is also serving other programs, the SSID's should be different.    Select the Destination Address and the Unproto Path. You can type your own Unproto Path or select one from the drop-down list. Iinitially just CQ as the Destination Address    with no Unproto Path is a good starting point. Select the Radio Ports you want the beacons to transmit on.    The number of Radio ports that are available is determined by the 'Comms Type' so you should select the Comms Type first.

If you have set the Comms Type as AGWPE or KISS and Object 1 is set to "weather station", APRS-Beacon will transmit the weather data to the Destination address set in 'Wx beacon destination'.

(If the Comms Type is TNC, weather transmissions will always use the 'normal' Destination address.)

Any changes you make to the Station Setup will take effect immediately - you don't need to shut down and restart APRS-Beacon.

APRS-Beacon Maps

Maps are loaded into the right-hand section of    APRS-Beacon by using either the File | Load Map menu option or the Load Map button on the Toolbar.

I've made the format of the maps compatible with those used by UI-View [2] so you can use the same maps.    Maps can be in any of the common graphics formats - BMP, GIF and JPG.    The scale of the maps doesn't matter - the only requirement is that the latitudes and longitudes are evenly spaced across the entire map.

Associated with each map is an information file. This is a simple text file with the same name as the map but with the extension .INF.    The format of this file is also compatible with UI-View :

First Line:	Latitude and Longitude of top left-hand corner.

Second Line:	Latitude and Longitude of bottom right-hand corner.

Third Line:	A "friendly" name for the map.

Note that latitudes and longitudes are expressed as Degrees.Minutes.DecimalMinutes.      To convert from Degrees.Minutes.Seconds    divide the Seconds by 60.      For example:

53.15.30N      =  53.15.5N      	    (because 30 ÷ 60 = .5)

Check the example file (NorthWest.inf) to see the format.

Please note that, although I've tried to make the .INF files compatible with UI-View, the error-checking in APRS-Beacon may not be as robust so it is quite likely that UI-View may be more tolerant of errors in the file.

Also note that APRS-Beacon will allow you to select any map file, whether it has an INF file or not,    so it is up to you to make sure that each map has its associated .INF file in the same folder as the map.

The Objects you create on a map are associated with that particular map. APRS-Beacon isn't smart enough to transfer Objects from map to map. Each map must have it's own Objects.

If you want to select a different map, this could leave the Objects "stranded" on the maps of other APRS stations so you may want to delete the Objects before you change maps. (See 'To Delete An Object')

Creating An Object

Once you have loaded a map, select which Object number (1, 2 or 3) you want to create by selecting the appropriate Tab in the Details Window of APRS-Beacon    and then click on the map at the location where you want the Object to appear.    The Latitude and Longitude will appear in the boxes in the Details Window.

Note that you cannot type text into these boxes - you can only select the latitude and longitude by clicking on the map.

Type a suitable identifier for the Object in the Identifier box (for example, RALLY or FETE etc) - a label will appear on the map as you type. Finally, select a suitable Symbol from the drop-down Symbol list.

APRS-Beacon only provides a limited number of Symbols which I selected for the program's intended use.

The second and third Objects can be created in the same way.

Set a Beacon Interval for each Object you have created. The minimum interval is 10 minutes and the maximum is 120 minutes.

You can also type an optional comment in the Beacon Comment box, which will be sent along with the position and symbol information when the beacon is transmitted.    If the first character is a % sign, the current local time will be inserted in its place when the beacon is sent.

Note that the Objects can obtain this information from    external text files as explained below.

To move an Object click on the Object on the map (the cursor should change to a cross) and then click on its new location.    If you click on an Object and then decide not to move it,    right-click anywhere else on the map or hit the 'ESC' key.

To Delelte an Object press the Delete button in the Details Window.    If you are connected to AGWPE or the TNC, you will be asked whether you want to send a transmission to notify other stations of the deletion. Comment text (if any) will not be sent. Note that you can't delete an Object while its timer is running - you need to press the Stop button first.

Transmitting the Beacon

Once the Object's details have been entered,    connect to AGWPE or the TNC by pressing the Connect button on the Toolbar or by clicking the Connect menu item.

You may now Send the Beacon immediately by pressing the Send Now button. Alternatively, press the Start button to allow the beacon to run automatically on the interval timer.    Press the Stop button to stop the interval timer.

There is a built-in separation of 5 seconds between transmissions. While APRS-Beacon is transmitting (or waiting to transmit), the cursor will change to an hourglass and you will not be able to make any changes in the Details window.

The Start and Send Now buttons can, optionally, send the comment either from the Beacon Comment box or from the external text file. The option is selected with the checkbox above the buttons.

If the comment is sent from the external file, the Beacon Comment box will be updated to show the actual text that was sent.

External Comment Files

APRS-Beacon can, optionally, send    comments from an external text file. The contents of the file are read immediately before the beacon is transmitted and can be updated at any time - even while APRS-Beacon is running. With a suitable program to create the file, this allows the beacon comment to be updated in real time even if the station is unattended.

The path and filenames of the external comment files are configured in APRS-Beacon under the Setup | Comment Files menu item. Once a comment file has been selected for an Object, you can choose whether or not to use it    by checking the checkbox    Use Comment File above the Start and Send Now buttons.

Format of the External Comment Files

The format of these files is similar to that for a weather station.    Each file    consists of two lines of text.    The first line is the local date and time the file was created and the second line is the comment. Any comment text over 40 characters will be ignored, as will any additional lines of text.

The format of the date and time can be any of the following:

Jun 12 1999    12:45		(Month dd    yyyy HH:MM)

12/06/1999 12:45		(dd/mm/yyyy HH:MM)

12/06/99 12:45			(dd/mm/yy    HH:MM)

Although the Beacon 'header' will always include the time in UTC,    this file-time can, optionally, be included in the Comment text by having a $ as the first character in the comment text.    (If the $ is not the first character on the line, it will just be sent as part of the text.)

If APRS-Beacon is unable to understand the date or time in the file, it will send the current local date and time instead.

If the first character is a % sign, the current local time will always be sent instead of the file-time.

A typical comment file which is to include the file-time in the transmitted comment may look something like this (without the dotted lines!):

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

Apr 16 2001 19:47

$ Radio Rally - doors open at 10am

- - -- - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

All the Objects may use the same Comment File or each may be configured to use different files. Note that there is no "expiry" date or time for the time stamp in the file.

Object 1 as a Weather Station

Object 1 can be used to transmit APRS-formatted weather data instead of an Object.    To use this option, select 'Weather Station'    from the Symbols drop-down list.    The check-box above the 'Start' and 'Send Now' buttons can then be checked to use a correctly-formatted weather data file or left un-checked, in which case the data can be entered 'manually'.    The format of the weather data file is similar to external comment files except the second line must be correctly formatted weather data of the form:-

DIR/SPDg000t000r000p000P000h00b00000

DIR/SPD    	is the wind direction and speed in mph.

g	is the speed of gusts sustained for at least 5 minutes.

t	is the air temperature in degrees F.

r	is the rainfall in the last hour in hundredths of an inch.

p	is the rainfall in the last 24 hours in hundredths of an inch.

P	is the rainfall since midnight in hundredths of an inch.

h	is the humidy in percent.

b	is the barometric pressure in tenths of a mBar.

To enter the weather data manually, press the additional 'Set data' button which appears when operating in 'weather station mode' and enter the appropriate weather data into the boxes.    You may leave some boxes blank if you wish. When you press the 'OK' button, the correctly-formatted weather data will automatically be entered into the box in the Details Window.    You should select whether you want to enter the data in English or Metric units before you enter any data (using the checkbox) because, although APRS-Beacon will convert between units, the mathematical 'rounding' process may lead to inaccuracies.

Closing Down APRS-Beacon

When APRS-Beacon is closed down, it will automatically disconnect from AGWPE and save the details of the map and any objects on it.    The same map and Objects (and the path to the Comment Files, if any) will be automatically loaded next time APRS-Beacon is started.

Files Created by APRS-Beacon

Two files are created and maintained automatically by APRS-Beacon.

APRSBeacon.ini  is a basic INI file which stores information about the size and position of the window and also a pointer to the folder and filename of the last-used Map file.    It also saves the names of the comment files and the Station and AGWPE / TNC details.

MapName.obd      saves information about the Objects on the map. It is saved in the same folder as the map.

Uninstalling APRS-Beacon

APRS-Beacon    is self-contained.    It does not install or require any external files (apart from the map files and the files it creates itself)    and does not change the system registry so, to uninstall APRS-Beacon, just remove the shortcut from the Desktop.

Configuring APRS-Beacon for your TNC

APRS-Beacon sends some common TNC commands to the TNC.    These commands tend to be fairly standard amongst different TNCs but, if you are unable to get APRS-Beacon to communicate with the TNC, these commands can be changed under the Setup | Communications menu.    Under the 'Comms Type', click the TNC Settings button.

'Command Code'    is the command you give the TNC to switch it to command-mode.    This is usually Ctrl-c    which has an ASCII value of $03 in Hexadecimal (or 3 in decimal).    The code may be entered in either Hex or decimal.    If the code is a Ctrl character, you can just hold the Ctrl key and hit the letter.

'Command Prompt'    is the command prompt the TNC sends after receiving the above 'Command' code. This is usually cmd:      If your TNC sends something different, edit this line.    Note that it is case-sensitive - cmd:    is not the same as CMD:

'SendPac Code'    is the keyboard character that makes the TNC transmit a packet when in Convers mode. It is usually the <Enter> key, which is $0D

in Hexadecimal (or 13 in decimal).    The code may be entered in Hex or decimal.    If the code is a Ctrl character, you can just hold the Ctrl key and hit the letter.    The <Enter> key is Ctrl-m    or just hit the <Enter> key!

'Beacon Timing'    is the command to set the TNC's beacon timing.    APRS-Beacon turns the TNC's beacon off because it takes care of its own beacon timing. BEACON EVERY 0    is the most common command for this.

'Converse Mode'    is the comand to switch the TNC into Conversational mode.    This is usually CONV.

'Connect Requests'    determines whether or not the TNC accepts Connect requests. APRS-Beacon turns this off for use on the un-connected APRS frequencies.    The default is CONOK OFF

I've provided a very simple terminal window in APRS-Beacon, which you can select under the View menu. From it, you should be able to issue simple commands to the TNC.

Credits and References

APRS is a Trademark    and the APRS protocols are copyright of APRS Engineering LLC.

President, Bob Bruninga, WB4APR.

GIF -to- BMP conversion routines for the maps, copyright Anders Melander, http://www.melander.dk/

[1]    AGWPE      Copyright George Rossopoulos (SV2AGW).

[2]    UI-View      Copyright Roger Barker (G4IDE) for Peak Systems.

APRS-Beacon is (c) Copyright John Owen (G4VWL).

john@vwlowen.demon.co.uk

http://www.vwlowen.demon.co.uk/

APRS-Beacon is freeware and may be freely distributed provided the complete     zip package is distibuted and no charge is made for it.

-------------------------------------------------------------------------

12 Jun 2001 version 1.1

18 Jun 2001    Version 1.2 - Support for TNCs added.

01 Jul 2001    Version 1.3 - Kiss suport added.

08 Jul 2001    Version 1.4 - Weather Station facilities added.